Jump to Spec

- UHD HDR Production Masters

- UHD SDR Production Masters

- HD SDR Production Masters

- UHD Dolby Vision Production Masters

- Near Field Printmaster 5.1 MXF

- Near Field Printmaster 2.0 MXF

- UHD HDR Textless Element Reel

- UHD SDR Textless Element Reel

- HD SDR Textless Element Reel

- Dolby Vision Textless Element Reel

- Studio Cut

- Audience Testing

- Theatrical Screeners

- Theatrical Locked Cut

- Theatrical Scripting Screener

- Locked Cut

- Visible Timecode Screeners

- Clean Screeners

- Dailies Proxy

- UHD Split Track Masters - 14ch

- HD Split Track Masters - 14ch

- HD Split Track Masters - 8ch

- UHD Split Track Masters - 16ch

- HD Split Track Masters - 16ch

- Digital Cinema Package - 4K Flat

- Digital Cinema Package - 4K Scope

- Digital Cinema Package - 2K Flat

- Digital Cinema Package - 2K Scope

- Dolby Cinema Digital Cinema Package - 4K Flat

- Dolby Cinema Digital Cinema Package - 4K Scope

- Theatrical Forced Narrative Subtitle - XML & Full Subtitle - XML

- Digital Cinema Package (DCP) Reel Screener

- Culver Theater HDR Digital Cinema Package - 4K Full

- UHD HDR Disclaimer Card

- UHD SDR Disclaimer Card

- HD SDR Disclaimer Cards

- Dolby Vision Disclaimer Cards

- Track Layouts

- Locked Cut Split Track Audio

- Nearfield WAV

- Near Field Pro Tools Session

- Near Field Asset Specs

- Near Field Dolby Atmos Printmaster

- Near Field Audio Description 2.0

- Near Field Audio Description 5.1 and 2.0

- Theatrical WAV

- Theatrical Pro Tools Session

- Theatrical Asset Specs

- Theatrical Dolby Atmos Printmaster

- Theatrical Audio Description

- Forced Narrative, Full SDH, Sans Forced Narrative SDH Subtitle, and Full Subtitles - ITT

- Forced Narrative, Full SDH, Sans Forced Narrative SDH, and Full Subtitle - CAP

- HDR-10 Metadata

- Dolby Vision Metadata

- Dolby Vision Metadata Example

- Original Capture Files (OCF)

- Non-Graded Archival Master (NAM)

- Digital Source Masters

- UHD HDR Digital Source Master

- UHD SDR Digital Source Master

- HD SDR Digital Source Master

- 4K Flat DCI-P3 Digital Source Master

- 4K Scope DCI-P3 Digital Source Master

- 2K Flat DCI-P3 Digital Source Master

- 2K Scope DCI-P3 Digital Source Master

- 4K Flat Digital Cinema Distribution Master

- 4K Scope Digital Cinema Distribution Master

- 2K Flat Digital Cinema Distribution Master

- 2K Scope Digital Cinema Distribution Master

- 4K Flat Dolby Vision Digital Cinema Distribution Master

- 4K Scope Dolby Vision Digital Cinema Distribution Master

Video

Production Masters

- Production Masters:

  - Do not include Color Bars, Leader, Slates, or Tones.

  - Contains 8 discrete Near Field audio channels.

    - 2 discrete channels are acceptable if 5.1 mix is not created.

  - 2.0 mixes must be provided as true stereo Left Only/Right Only (Lo/Ro) audio.

  - Start of File (SOF) and First Frame of Program (FFOP) both start at hour 0.

  - Last Frame of Program (LFOP) and End of File (EOF) will be the last frame of video and/or audio.

  - Safe Action: Follow SMPTE ST 2046-1:2009 - Specifications for Safe Action & Safe Title Areas for Television.

  - On some programs you will be required to provide an additional version. See Edits instructions for more info.

  - ProRes files encoded as full range YCbCr will be rejected during QC because it generates clipped code values.

    - Resolve: Use Auto data levels

    - Transkoder: Use default range

- Types of Production Masters:

  - Texted

  - Non-Subtitled

  - Textless - Created only when 30% or more of the program is texted.

- Program Layout for all files unless specified otherwise:

SOF EOF

Program

FFOP LFOP

00:00:00:00

Documentary Style/Unscripted Content

- For Documentary Style/Unscripted content where on-screen graphics/chyron are present throughout the program, the preference is to provide a Textless Production Master.

UHD HDR Production Masters

| Asset Spec | UHD HDR Production Master 4444XQ - Non-Subtitled - 5.1 and 2.0 UHD HDR Production Master 4444XQ - Texted - 5.1 and 2.0 UHD HDR Production Master 4444XQ - Textless - 5.1 and 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ (Required) Apple ProRes 422 HQ (Supported) |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Color encoding primaries: Rec.2020 Mastering display primaries: P3 D65 white point |
| Maximum Luminance | 1000 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Last Frame of Program | Last frame of video and/or audio. |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 or 2 |
| Loudness Target | -27 LKFS ± 2 LU |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping | 8 Channel |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | Left Only |
| Channel 8 | Right Only |

UHD SDR Production Masters

| Asset Spec | UHD SDR Production Master 4444XQ - Non-Subtitled - 5.1 and 2.0 UHD SDR Production Master 4444XQ - Texted - 5.1 and 2.0 UHD SDR Production Master 4444XQ - Textless - 5.1 and 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ (Required) Apple ProRes 422 HQ (Supported) |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 or 2 |
| Loudness Target | -27 LKFS ± 2 LU |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping | 8 Channel |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | Left Only |
| Channel 8 | Right Only |

HD SDR Production Masters

| Asset Spec | HD SDR Production Master 4444XQ - Non-Subtitled - 5.1 and 2.0 HD SDR Production Master 4444XQ - Texted - 5.1 and 2.0 HD SDR Production Master 4444XQ - Textless - 5.1 and 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ (Required) Apple ProRes 422 HQ (Supported) |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 or 2 |
| Loudness Target | -27 LKFS ± 2 LU |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping | 8 Channel |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | Left Only |
| Channel 8 | Right Only |

UHD Dolby Vision Production Masters

- Nearfield 5.1 and 2.0 audio to be sent as separate MXF files.

- Dolby Vision Metadata is embedded in the .mxf file and also provided as a sidecar asset.

| Asset Spec | UHD Dolby Vision Production Master - Non-Subtitled UHD Dolby Vision Production Master - Texted UHD Dolby Vision Production Master - Textless |
| --- | --- |
| File Extension | .mxf (Material Exchange Format) |
| Video Container | MXF |
| Video Codec | JPEG 2000 |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Color encoding primaries: Rec.2020 Mastering display primaries: P3 D65 white point |
| Color Range | RGB |
| JPEG 2000 Profile | Main 6, Sub 3, RGB444 12Bit, QE2 Full Range, IMF Lossy Profile, App 2e+ 2020, Color Space Color7: Rec2020 HDR, Mastering Display P3 |
| Maximum Luminance | 1000 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Dolby Vision Metadata | CMv4.0 embedded into MXF wrapper |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

Near Field Printmaster 5.1 MXF

| Asset Spec | Near Field Printmaster 5.1 MXF |
| --- | --- |
| File Extension | .mxf |
| Audio Container | MXF (OP1a) |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 6 |
| Monitoring Levels | 79dB - 82dB |
| Loudness Target | -27 LKFS ± 2 LU |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Channel Mapping | 6 Channel |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |

Near Field Printmaster 2.0 MXF

| Asset Spec | Near Field Printmaster 2.0 MXF |
| --- | --- |
| File Extension | .mxf |
| Audio Container | MXF (OP1a) |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 2 |
| Monitoring Levels | 79dB - 82dB |
| Loudness Target | -27 LKFS ± 2 LU |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Channel Mapping | 2 Channel |
| Channel 1 | Left Only |
| Channel 2 | Right Only |

Textless Element Reels

- Do not include Color Bars, Leader, or Tones.

- Textless Element Reels do not include any audio.

- Slate at SOF to include:

  - Label: [UHD or HD] [HDR or SDR] Textless Element Reel

  - Show Title:

  - Show Code:

  - Episode Title: [If applicable]

  - Episode #: [If applicable]

  - Run Time: Measure from FFOP to LFOP. Do not include slate and black.

  - Date Created:

  - Audio Config: Not Applicable

- Textless Elements to be provided in sequential order, as they pertain to the program.

- Textless Elements must be frame accurate to Production Master.

  - Example: If the Production Master first Textless Element is at 00:02:36:12, provide textless shot at 00:00:10:12 on the Textless Element Reel.

  - The timecodes will not be the same but the frame numbers will be.

  - Black between textless shots will be between 2-3 seconds due to the frame requirement.

- In/out points to be set on hard cuts, where no dissolves or transitions are present.

- Program Layout:

| SOF |  | EOF |
| --- | --- | --- |
| Slate | Black | Textless Elements |
| 8 Sec | 2 Sec | FFOP LFOP |
| 00:00:00:00 | 00:00:08:00 | 00:00:10:00 |

Documentary Style/Unscripted Content

- For Documentary Style/Unscripted content where on-screen graphics/chyron are present throughout program, the preference is to provide a Textless Production Master. A Textless Element Reel is not required when a Textless Production Master is created.

UHD HDR Textless Element Reel

| Asset Spec | UHD HDR Textless Element Reel 4444XQ |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ (Required) Apple ProRes 422 HQ (Supported) |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Color encoding primaries: Rec.2020 Mastering display primaries: P3 D65 white point |
| Maximum Luminance | 1000 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

UHD SDR Textless Element Reel

| Asset Spec | UHD SDR Textless Element Reel 4444XQ |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ Apple ProRes 422 HQ (Supported) |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

HD SDR Textless Element Reel

| Asset Spec | HD SDR Textless Element Reel 4444XQ |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ Apple ProRes 422 HQ (Supported) |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Color Range | RGB |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

Dolby Vision Textless Element Reel

- Dolby Vision Metadata is embedded in the .mxf file and also provided as a sidecar asset.

| Asset Spec | Dolby Vision Textless Element Reel |
| --- | --- |
| File Extension | .mxf (Material Exchange Format) |
| Video Container | MXF |
| Video Codec | JPEG 2000 |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Color encoding primaries: Rec.2020 Mastering display primaries: P3 D65 white point |
| Color Range | RGB |
| JPEG 2000 Profile | Main 6, Sub 3, RGB444 12Bit, QE2 Full Range, IMF Lossy Profile, App 2e+ 2020, Color Space Color7: Rec2020 HDR, Mastering Display P3 |
| Maximum Luminance | 1000 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Dolby Vision Metadata | CMv4.0 embedded into MXF wrapper |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

Studio Cut

Studio Cuts

- Types of Studio Cuts:

  - Texted

  - With ECR Subtitles (English Subtitled): Includes English Creative Review subtitles for non-English content. Primary language Forced Narrative subtitles must be burned-in to video (if applicable).

- Audio must be interleaved.

| Asset Spec | Studio Cut - Texted - 2.0 Studio Cut - With ECR Subtitles (English Subtitled) - 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | AVC (H.264) |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Color Range | YUV |
| Chroma Bit Depth | 8-bit |
| Chroma Sub Sampling | 4:2:0 |
| Maximum Data Rate | 10,000 kbps |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Visible Timecode | Upper Right |
| Watermarking | Lower Third Center - Property of [Production Co.], [Individual Recipient Name], [Date] Opacity set at 25%. |
| Audio |  |
| Audio Codec | AAC-LC |
| Audio Sample Rate | 48kHz |
| Audio Channels | 2 |
| Channel Mapping |  |
| Channel 1 | Left Only |
| Channel 2 | Right Only |

Audience Testing

- Consult with your Post Executive to determine if branding should be removed/hidden and if watermarks are required.

- Program Layout:

| SOF | EOF |
| --- | --- |
| Program | Black |
| FFOP LFOP | 2 Sec |
| 00:00:00:00 |  |

| Asset Spec | Audience Testing - Texted - 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | AVC (H.264) |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Color Range | YUV |
| Chroma Bit Depth | 8-bit |
| Chroma Sub Sampling | 4:2:0 |
| Maximum Data Rate | 10,000 kbps |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| End of File | Remove End Credit Bed and add 2 seconds of black after program ends. |
| VFX Marker Descriptions | Burned-in to picture. |
| Audio |  |
| Audio Codec | AAC |
| Audio Sample Rate | 48kHz |
| Audio Bit Rate | 192 kpbs |
| Audio Channels | 2 |
| Channel Mapping |  |
| Channel 1 | Left Only |
| Channel 2 | Right Only |

Theatrical Screeners

- Each Reel must start with unique Reel TC (R1 = 01:00:00:00, R2 = 02:00:00:00, etc.)

- Program Layout (24fps Example):

| SOF EOF |  |  |  |  |
| --- | --- | --- | --- | --- |
| Leader | 2 Pop | Program | Black | 2 Pop |
| 8 Sec | 1 Frame | FFOP LFOP | 47 Frames | 1 Frame |
| 01:00:00:00 | 01:00:06:00 | 01:00:08:00 |  |  |

Theatrical Locked Cut

| Asset Spec | Theatrical Locked Cut - 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | AVC (H.264) |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Color Range | YUV |
| Chroma Bit Depth | 8-bit |
| Chroma Sub Sampling | 4:2:0 |
| Maximum Data Rate | 10,000 kbps |
| Frame Rate | 24 |
| Scan Type | Progressive |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| Visible Timecode | Upper Right (Preferably in the matte) |
| Visible Feet & Frames | Upper Left (Preferably in the matte) |
| Watermarking | Opacity set at 30% Upper Third Center: Property of Amazon MGM Studios [Name of QC Vendor] Lower Third Center: Version # - Do not obscure Forced Narrative/Subtitles |
| Audio |  |
| Audio Codec | AAC-LC |
| Audio Sample Rate | 48kHz |
| Audio Bit Rate | 192 kbps |
| Audio Channels | 2 |
| Channel Mapping |  |
| Channel 1 | Left Only (Prelim) |
| Channel 2 | Right Only (Prelim) |

Theatrical Scripting Screener

Scripting Vendor also requires:

- Production/Line Scripts

- Cast List

| Asset Spec | Theatrical Scripting Screener - 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | AVC (H.264) |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Color Range | YUV |
| Chroma Bit Depth | 8-bit |
| Chroma Sub Sampling | 4:2:0 |
| Maximum Data Rate | 10,000 kbps |
| Frame Rate | 24 |
| Scan Type | Progressive |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| Visible Timecode | Upper Right (Preferably in the matte) |
| Visible Feet & Frames | Upper Left (Preferably in the matte) |
| Watermarking | Opacity set at 30% Upper Third Center: Property of Amazon MGM Studios [Name of Script Vendor] Lower Third Center: Version # - Do not obscure Forced Narrative/Subtitles |
| Audio |  |
| Audio Codec | AAC-LC |
| Audio Sample Rate | 48kHz |
| Audio Bit Rate | 192 kbps |
| Audio Channels | 2 |
| Channel Mapping |  |
| Channel 1 | Mono Dialogue (Prelim or Final) |
| Channel 2 | Mono Printmaster (Prelim or Final) |

Locked Cut

Locked Cut

- Types of Locked Cuts:

  - Texted

  - With ECR Subtitles (English Subtitled): Includes English Creative Review subtitles for non-English content. Primary language Forced Narrative subtitles must be burned-in to video (if applicable).

- Audio must be interleaved.

- Locked Cut Split Track Audio must be delivered with Locked Cut screener.

| Asset Spec | Locked Cut - Texted - 2.0 Locked Cut - With ECR Subtitles (English Subtitled) - 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | AVC (H.264) |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Color Range | YUV |
| Chroma Bit Depth | 8-bit |
| Chroma Sub Sampling | 4:2:0 |
| Maximum Data Rate | 10,000 kbps |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Visible Timecode | Upper Right |
| Watermarking | Lower Third Center - Property of [Production Co.], [Individual Recipient Name], [Date] |
| Audio |  |
| Audio Codec | AAC-LC |
| Audio Sample Rate | 48kHz |
| Audio Bit Rate | 192 kbps |
| Audio Channels | 2 |
| Channel Mapping |  |
| Channel 1 | Left Only |
| Channel 2 | Right Only |

Screeners

- Types of Visible Timecode and Clean Screeners:

  - Texted

  - With English Creative Review (ECR) Subtitles, only for non-English content. Forced Narrative subtitles must also be burned-in (if applicable).

- Audio must be interleaved.

Visible Timecode Screeners

| Asset Spec | VisTC Screener - Texted - 2.0 VisTC Screener - With ECR Subtitles (English Subtitled) - 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | AVC (H.264) |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Color Range | YUV |
| Chroma Bit Depth | 8-bit |
| Chroma Sub Sampling | 4:2:0 |
| Maximum Data Rate | 10000 kbps |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Visible Timecode | Upper Right |
| Watermarking | Lower Third - Property of [Production Co.] Opacity set at 25%. |
| Audio |  |
| Audio Codec | AAC |
| Audio Sample Rate | 48kHz |
| Audio Bit Rate | 192 kbps |
| Audio Channels | 2 |
| Channel Mapping |  |
| Channel 1 | Left Only |
| Channel 2 | Right Only |

Clean Screeners

| Asset Spec | Clean Screener - Texted - 2.0 Clean Screener - With ECR Subtitles (English Subtitled) - 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | AVC (H.264) |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Color Range | YUV |
| Chroma Bit Depth | 8-bit |
| Chroma Sub Sampling | 4:2:0 |
| Maximum Data Rate | 10000 kbps |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Audio |  |
| Audio Codec | AAC |
| Audio Sample Rate | 48kHz |
| Audio Bit Rate | 192 kbps |
| Audio Channels | 2 |
| Channel Mapping |  |
| Channel 1 | Left Only |
| Channel 2 | Right Only |

Dailies Proxy

| Asset Specs | Dailies Proxy - 2.0 |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | AVC (H.264) |
| Resolution Height | 720 |
| Resolution Width | 1280 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Color Range | YUV |
| Chroma Bit Depth | 8-bit |
| Chroma Sub Sampling | 4:2:0 |
| Maximum Data Rate | 2400 kbps |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Visible Timecode | Upper Right |
| Watermarking | Lower Third Center - Property of [Production Co.], [Individual Recipient Name], [Date] Opacity set at 25%. |
| Audio |  |
| Audio Codec | AAC-LC |
| Audio Sample Rate | 48kHz |
| Audio Channels | 4 |
| Channel Mapping |  |
| Channel 1 | Production Sound |
| Channel 2 | Production Sound |

Split Track Masters

- Slate at SOF to include:

  - Label: [UHD or HD] SDR Split Track Master

  - Show Title:

  - Show Code:

  - Episode Title: [If applicable]

  - Episode #: [If applicable]

  - Run Time: Measure from FFOP to LFOP. Do not include slate and black.

  - Date Created:

  - Audio Config:

- Loudness Target only applies to Ch.1-8 audio.

- Forced Narrative subtitles must be burned into picture (when applicable).

- Textless at tail must include ELMT reel sequences and coverage for Forced Narrative (FN) events in program, where applicable.

- For content where 30% or more of the program is texted, Production Company should provide a Texted Split Track Master in addition to a Fully Textless Split Track Master.

- Program Layout:

| Split Track Master |  |  |  |  |
| --- | --- | --- | --- | --- |
| SOF EOF |  |  |  |  |
| Slate | Audio/Visual 2 Pop | Program | Black | Textless Element Reel |
| 8 Sec | 2 Sec | FFOP - LFOP | 10 Sec |  |
| 00:59:50:00 | 00:59:58:00 | 01:00:00:00 |  |  |

- When delivering Texted Split Track Master and Fully Textless Split Track Master the Textless Element Reel does not need to be included.

| Textless Split Track Master |  |  |
| --- | --- | --- |
| SOF EOF |  |  |
| Slate | 2 Pop | Program |
| 8 Sec | 2 Sec | FFOP - LFOP |
| 00:59:50:00 | 00:59:58:00 | 01:00:00:00 |

Documentary Style/Unscripted

- For Documentary Style/Unscripted content where on-screen graphics/chyron are present throughout program, the preference is to provide a Textless Split Track Master.

- Documentary Style/Unscripted Split Track Masters require a different audio configuration which allows for up to 16 channels of audio.

- If narration doesn't exist for channels 10, 15, and 16 then the channel must include MOS audio.

UHD Split Track Masters - 14ch

| Asset Spec | UHD Split Track Master 422HQ - 14ch UHD Textless Split Track Master 422HQ - 14ch |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 422 HQ |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 10-bit |
| Chroma Sub Sampling | 4:2:2 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:59:50:00 |
| Slate | 8 Seconds |
| 2 Pop | 2 Seconds |
| First Frame of Program | 01:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 14 |
| Loudness Target | -27 LKFS ± 2 LU (Applies to Printmaster Ch.1-8 only) |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | Left Only |
| Channel 8 | Right Only |
| Channel 9 | Left Dialogue |
| Channel 10 | Right Dialogue |
| Channel 11 | Left Effects |
| Channel 12 | Right Effects |
| Channel 13 | Left Music |
| Channel 14 | Right Music |

HD Split Track Masters - 14ch

| Asset Spec | HD Split Track Master 422HQ - 14ch HD Textless Split Track Master 422HQ - 16ch |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 422 HQ |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 10-bit |
| Chroma Sub Sampling | 4:2:2 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:59:50:00 |
| Slate | 8 Seconds |
| 2 Pop | 2 Seconds |
| First Frame of Program | 01:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 14 |
| Loudness Target | -27 LKFS ± 2 LU (Applies to Printmaster Ch.1-8 only) |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | Left Only |
| Channel 8 | Right Only |
| Channel 9 | Left Dialogue |
| Channel 10 | Right Dialogue |
| Channel 11 | Left Effects |
| Channel 12 | Right Effects |
| Channel 13 | Left Music |
| Channel 14 | Right Music |

HD Split Track Masters - 8ch

| Asset Spec | HD Split Track Master 422HQ - 8ch HD Textless Split Track Master 422HQ - 8ch |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 422 HQ |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 10-bit |
| Chroma Sub Sampling | 4:2:2 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:59:50:00 |
| Slate | 8 Seconds |
| 2 Pop | 2 Seconds |
| First Frame of Program | 01:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 14 |
| Loudness Target | -27 LKFS ± 2 LU (Applies to Printmaster Ch.1-8 only) |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left Only |
| Channel 2 | Right Only |
| Channel 3 | Left Dialogue |
| Channel 4 | Right Dialogue |
| Channel 5 | Left Effects |
| Channel 6 | Right Effects |
| Channel 7 | Left Music |
| Channel 8 | Right Music |

Documentary Style/Unscripted Split Track Masters

UHD Split Track Masters - 16ch

| Asset Spec | UHD Split Track Master 422HQ - 16ch UHD Textless Split Track Master 422HQ - 16ch |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 422 HQ |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 10-bit |
| Chroma Sub Sampling | 4:2:2 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:59:50:00 |
| Slate | 8 Seconds |
| 2 Pop | 2 Seconds |
| First Frame of Program | 01:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 16 |
| Loudness Target | -27 LKFS ± 2 LU (Applies to Printmaster Ch.1-8 only) |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | Left Only |
| Channel 8 | Right Only |
| Channel 9 | Dialogue (Production Sound) |
| Channel 10 | Narration or MOS (if applicable) |
| Channel 11 | Left Effects |
| Channel 12 | Right Effects |
| Channel 13 | Left Music |
| Channel 14 | Right Music |
| Channel 15 | Left Mix Minus Narration or MOS (If applicable) |
| Channel 16 | Right Mix Minus Narration or MOS (If applicable) |

HD Split Track Masters - 16ch

| Asset Spec | HD Split Track Master 422HQ - 16ch UHD Split Track Master 422HQ - 16ch |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 422 HQ |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 10-bit |
| Chroma Sub Sampling | 4:2:2 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:59:50:00 |
| Slate | 8 Seconds |
| 2 Pop | 2 Seconds |
| First Frame of Program | 01:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 16 |
| Loudness Target | -27 LKFS ± 2 LU (Applies to Printmaster Ch.1-8 only) |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | Left Only |
| Channel 8 | Right Only |
| Channel 9 | Dialogue (Production Sound) |
| Channel 10 | Narration or MOS (If applicable) |
| Channel 11 | Left Effects |
| Channel 12 | Right Effects |
| Channel 13 | Left Music |
| Channel 14 | Right Music |
| Channel 15 | Left Mix Minus Narration or MOS (If applicable) |
| Channel 16 | Right Mix Minus Narration or MOS (If applicable) |

Theatrical

Digital Cinema Package (DCP)

- All video, audio, and subtitles must be in sync.

- DCP's must be encrypted and a DKDM must be created for each DCP.

- Follow Deluxe - Recommended Guidelines for Digital Cinema Source and DCP Content Delivery.

- Follow InterSociety Digital Cinema Forum Naming Convention.

- Logos

  - Logos must match benchmark logos and may not be altered.

- DCP Formats:

  - Interop (IOP)

    - If the native frame rate is not 24 fps then it must be converted to 24 fps.

  - SMPTE

    - 24 fps is the standard requirement. Please contact your Amazon Post/Delivery Exec if the DI frame rate is different than 24 fps. Utilized for Dolby Vision DCP’s and/or titles with Dolby Atmos.

- Consult with your Delivery Exec to determine which DCP format is required and how it should be packaged.

- DCI Container Aspect Ratios:

  - Flat (1.85) containers are for titles with an aspect ratio less than 2.30:1.

  - Scope (2.39) containers are for titles with an aspect ratio greater than or equal to 2.30:1.

- Package Types:

  - Original Version (OV) always contains an entire program/package.

  - Version File (VF) usually contains a partial feature (e.g.: subtitles, alternate languages, hold cards...) and must be loaded together with the OV track files in order to play the program correctly.

- 7.1/Dolby Atmos audio must be delivered as a VF. Dolby Atmos Audio VF package will contain a backup 5.1 or 7.1 audio configuration.

- Composition Playlist (CPL) in and out points are to be set to play a continuous longplay program. Slate, leader, and 2 pops should not play.

- Program Layout (24fps Example):

| SOF EOF |  |  |  |  |
| --- | --- | --- | --- | --- |
| Leader | 2 Pop | Program | Black | 2 Pop |
| 8 Sec | 1 Frame | FFOP LFOP | 47 Frames | 1 Frame |
| 01:00:00:00 | 01:00:06:00 | 01:00:08:00 |  |  |

Series

- Head logo must be tied to the head of the episode in longplay format.

- Tail logo must be tied to the tail of the episode in longplay format.

Amazon Original Movies (AOM)

- Please check with your AMS Post Exec if head logo should be on Reel 0 by itself.

- Please check with your AMS Post Exec if tail logo should be on last Reel by itself

- Interop DCP’s with image based Forced Narrative subtitle .xml’s need to be included in both OV and 7.1 VF packages (if applicable). Subtitles will not link between Interop OV and VF packages.

- Each Reel must start with unique Reel TC (R1 = 01:00:00:00, R2 = 02:00:00:00, etc.)

Digital Cinema Package - 4K Flat

| Asset Spec | Digital Cinema Package - 4K Flat |
| --- | --- |
| File Extension | .mxf |
| DCP Format | Interop (IOP) or SMPTE |
| Video Container | MXF |
| Video Codec | JPEG 2000 |
| Resolution Height | 2160 |
| Resolution Width | 3996 |
| Container Aspect Ratio | Flat (1.85) |
| Color Space | XYZ |
| Maximum Luminance | 48 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Maximum Data Rate | 245 Mbps (VBR) |
| Transfer Function | Gamma 2.6 |
| Frame Rate | 24 |
| Scan Type | Progressive |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| End of File | 2 seconds of black and tail pop after LFOP. |
| Audio |  |
| Audio Container | MXF |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Monitoring Levels | 85dB |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | MOS |
| Channel 8 | MOS |

Digital Cinema Package - 4K Scope

| Asset Spec | Digital Cinema Package - 4K Scope |
| --- | --- |
| File Extension | .mxf |
| DCP Format | Interop (IOP) or SMPTE |
| Video Container | MXF |
| Video Codec | JPEG 2000 |
| Resolution Height | 1716 |
| Resolution Width | 4096 |
| Container Aspect Ratio | Scope (2.39) |
| Color Space | XYZ |
| Maximum Luminance | 48 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Maximum Data Rate | 245 Mbps (VBR) |
| Transfer Function | Gamma 2.6 |
| Frame Rate | 24 |
| Scan Type | Progressive |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| End of File | 2 seconds of black and tail pop after LFOP. |
| Audio |  |
| Audio Container | MXF |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Monitoring Levels | 85dB |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | MOS |
| Channel 8 | MOS |

Digital Cinema Package - 2K Flat

| Asset Spec | Digital Cinema Package - 2K Flat |
| --- | --- |
| File Extension | .mxf |
| DCP Format | Interop (IOP) or SMPTE |
| Video Container | MXF |
| Video Codec | JPEG 2000 |
| Resolution Height | 1080 |
| Resolution Width | 1998 |
| Container Aspect Ratio | Flat (1.85) |
| Color Space | XYZ |
| Maximum Luminance | 48 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Maximum Data Rate | 245 Mbps (VBR) |
| Transfer Function | Gamma 2.6 |
| Frame Rate | 24 |
| Scan Type | Progressive |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| End of File | 2 seconds of black and tail pop after LFOP. |
| Audio |  |
| Audio Container | MXF |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Monitoring Levels | 85dB |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | MOS |
| Channel 8 | MOS |

Digital Cinema Package - 2K Scope

| Asset Spec | Digital Cinema Package - 2K Scope |
| --- | --- |
| File Extension | .mxf |
| DCP Format | Interop (IOP) or SMPTE |
| Video Container | MXF |
| Video Codec | JPEG 2000 |
| Resolution Height | 858 |
| Resolution Width | 2048 |
| Container Aspect Ratio | Scope (2.39) |
| Color Space | XYZ |
| Maximum Luminance | 48 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Maximum Data Rate | 245 Mbps (VBR) |
| Transfer Function | Gamma 2.6 |
| Frame Rate | 24 |
| Scan Type | Progressive |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| End of File | 2 seconds of black and tail pop after LFOP. |
| Audio |  |
| Audio Container | MXF |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Monitoring Levels | 85dB |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | MOS |
| Channel 8 | MOS |

Dolby Cinema Digital Cinema Package

- Must be created in SMPTE DCP format.

Dolby Cinema Digital Cinema Package - 4K Flat

| Asset Spec | Dolby Cinema Digital Cinema Package - 4K Flat |
| --- | --- |
| File Extension | .mxf |
| DCP Format | SMPTE |
| Video Container | MXF |
| Video Codec | JPEG 2000 |
| Resolution Height | 2160 |
| Resolution Width | 3996 |
| Container Aspect Ratio | Flat (1.85) |
| Color Space | DCI-P3 |
| Maximum Luminance | 108 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Maximum Data Rate | 500 Mbps (VBR) |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Scan Type | Progressive |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| End of File | 2 seconds of black and tail pop after LFOP. |
| Audio |  |
| Audio Container | MXF |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Monitoring Levels | 85dB |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | MOS |
| Channel 8 | MOS |

Dolby Cinema Digital Cinema Package - 4K Scope

| Asset Spec | Dolby Cinema Digital Cinema Package - 4K Scope |
| --- | --- |
| File Extension | .mxf |
| DCP Format | SMPTE |
| Video Container | MXF |
| Video Codec | JPEG 2000 |
| Resolution Height | 1716 |
| Resolution Width | 4096 |
| Container Aspect Ratio | Scope (2.39) |
| Color Space | DCI-P3 |
| Maximum Luminance | 108 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Maximum Data Rate | 500 Mbps (VBR) |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Scan Type | Progressive |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| End of File | 2 seconds of black and tail pop after LFOP. |
| Audio |  |
| Audio Container | MXF |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Monitoring Levels | 85dB |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | MOS |
| Channel 8 | MOS |

Theatrical Dolby Atmos Printmaster

Theatrical Forced Narrative Subtitle - XML & Full Subtitle - XML

- Interop DCP subtitles must conform to the Texas Instruments DCinema Subtitle Specification V1.1.

- SMPTE DCP subtitles must conform to the SMPTE ST 428-7:2014 DCDM Subtitle specification.

| Asset Spec | Forced Narrative Subtitle - XML Full Subtitle - XML |
| --- | --- |
| File Extension | .xml |
| Timed Text Format | XML |
| Encoding | UTF-8 |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Minimum Text Duration | 5/6 of a second (20 frames for 24 fps) |
| Maximum Text Duration | 7 seconds per subtitle event. |

Distribution Key Delivery Message (DKDM)

- A DKDM with an open 5 year window must be provided with each DCP. The leaf certificate must be targeted to the to the distribution vendor.

Digital Cinema Package (DCP) Reel Screener

- Screeners are reel based and must match DCP. The number of DCP Reel screeners will match the total number of reels in the DCP.

| Asset Spec | Digital Cinema Package Reel Screener |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | AVC (H.264) |
| Resolution Height | 720 |
| Resolution Width | 1280 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Color Range | YUV |
| Chroma Bit Depth | 8-bit |
| Chroma Sub Sampling | 4:2:0 |
| Maximum Data Rate | 2400 kbps |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Scan Type | Progressive |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| End of File | 2 seconds of black and tail pop after LFOP. |
| Visible Timecode | Upper Right |
| Watermarking | Lower Third Center - Property of [Production Co.] Opacity set at 25%. |
| Audio |  |
| Audio Codec | AAC-LC |
| Audio Sample Rate | 48kHz |
| Audio Channels | 6 |
| Channel Mapping | 6 Channel |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |

Culver Theater HDR Digital Cinema Package - 4K Full

- Preference is to deliver as a single reel.

- Theatrical Dolby Atmos Printmaster must be delivered as a VF package (if applicable).

| Asset Spec | Culver Theater Digital Cinema Package - 4K Full |
| --- | --- |
| File Extension | .mxf |
| DCP Format | SMPTE |
| Video Container | MXF |
| Video Codec | JPEG 2000 |
| Resolution Height | 2160 |
| Resolution Width | 4096 |
| Active Image Resolution | 3840x2160 |
| Container Aspect Ratio | Full (1.90) |
| Color Space | XYZ |
| Maximum Luminance | 108 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Maximum Data Rate | 245 Mbps (VBR) |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 24 |
| Scan Type | Progressive |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| End of File | 2 seconds of black and tail pop after LFOP. |
| Audio |  |
| Audio Container | MXF |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Monitoring Levels | 85dB |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Channel Mapping |  |
| Channel 1 | Left |
| Channel 2 | Right |
| Channel 3 | Center |
| Channel 4 | LFE |
| Channel 5 | Left Surround |
| Channel 6 | Right Surround |
| Channel 7 | MOS |
| Channel 8 | MOS |

Disclaimer Cards

- Disclaimer Cards require 8 channels of MOS audio to facilitate AS Distribution workflows.

- 1/2 Second fade-in

- 1/2 Second fade-out

- White text over black background.

- Please consult with your AS Post Executive to determine length.

- A disclaimer card is required for every type of dynamic range applicable (HDR, SDR, and Dolby Vision).

UHD HDR Disclaimer Card

| Asset Spec | UHD HDR Disclaimer Card 4444XQ |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.2020 Container with D65 white point and constrained to a DCI-P3 gamut |
| Maximum Luminance | 1000 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Channel Mapping | 8 Channel |
| Channel 1 | MOS |
| Channel 2 | MOS |
| Channel 3 | MOS |
| Channel 4 | MOS |
| Channel 5 | MOS |
| Channel 6 | MOS |
| Channel 7 | MOS |
| Channel 8 | MOS |

UHD SDR Disclaimer Card

| Asset Spec | UHD SDR Disclaimer Card 4444XQ |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Channel Mapping | 8 Channel |
| Channel 1 | MOS |
| Channel 2 | MOS |
| Channel 3 | MOS |
| Channel 4 | MOS |
| Channel 5 | MOS |
| Channel 6 | MOS |
| Channel 7 | MOS |
| Channel 8 | MOS |

HD SDR Disclaimer Cards

| Asset Spec | HD SDR Disclaimer Card 4444XQ |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Color Range | RGB |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Channel Mapping | 8 Channel |
| Channel 1 | MOS |
| Channel 2 | MOS |
| Channel 3 | MOS |
| Channel 4 | MOS |
| Channel 5 | MOS |
| Channel 6 | MOS |
| Channel 7 | MOS |
| Channel 8 | MOS |

Dolby Vision Disclaimer Cards

| Asset Spec | UHD Dolby Vision Disclaimer Card |
| --- | --- |
| File Extension | .mxf (Material Exchange Format) |
| Video Container | MXF |
| Video Codec | JPEG 2000 |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Color encoding primaries: Rec.2020 Mastering display primaries: P3 D65 white point |
| Color Range | RGB |
| JPEG 2000 Profile | Main 6, Sub 3, RGB444 12Bit, QE2 Full Range, IMF Lossy Profile, App 2e+ 2020, Color Space Color7: Rec2020 HDR, Mastering Display P3 |
| Maximum Luminance | 1000 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Dolby Vision Metadata | CMv4.0 embedded into MXF wrapper |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

UHD HDR Dub Cards

| Asset Spec | UHD HDR Dub Cards 422HQ UHD HDR Dub Cards 4444XQ |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ (Required) Apple ProRes 422 HQ (Supported) |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.2020 Container with D65 white point and constrained to a DCI-P3 gamut |
| Maximum Luminance | 1000 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Channel Mapping | 8 Channel |
| Channel 1 | MOS |
| Channel 2 | MOS |
| Channel 3 | MOS |
| Channel 4 | MOS |
| Channel 5 | MOS |
| Channel 6 | MOS |
| Channel 7 | MOS |
| Channel 8 | MOS |

UHD SDR Dub CardS

| Asset Spec | UHD SDR Dub Cards 422HQ UHD SDR Dub Cards 4444XQ |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ (Required) Apple ProRes 422 HQ (Supported) |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Channel Mapping | 8 Channel |
| Channel 1 | MOS |
| Channel 2 | MOS |
| Channel 3 | MOS |
| Channel 4 | MOS |
| Channel 5 | MOS |
| Channel 6 | MOS |
| Channel 7 | MOS |
| Channel 8 | MOS |

HD SDR Dub Cards

| Asset Spec | HD SDR Dub Cards 422HQ HD SDR Dub Cards 4444XQ |
| --- | --- |
| File Extension | .mov (QuickTime) |
| Video Container | MOV |
| Video Codec | Apple ProRes 4444 XQ (Required) Apple ProRes 422 HQ (Supported) |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Pixel Aspect Ratio | 1:1 |
| Color Space | Rec.709 |
| Color Range | RGB |
| Maximum Luminance | 100 |
| Chroma Bit Depth | 12-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | BT.1886 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Scan Type | Progressive |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Audio |  |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Channel Mapping | 8 Channel |
| Channel 1 | MOS |
| Channel 2 | MOS |
| Channel 3 | MOS |
| Channel 4 | MOS |
| Channel 5 | MOS |
| Channel 6 | MOS |
| Channel 7 | MOS |
| Channel 8 | MOS |

Audio

Audio Asset Specs

- Audio must conform to final picture.

- Each .wav file must be flattened.

- Discrete .wav files required per channel. Interleaved audio is not acceptable.

- 2.0 mixes must be provided as true stereo Left Only/Right Only (Lo/Ro) audio.

- Do not include leader, slate, 2-pop, or post roll.

- All Dialogue, Music, and Effects stems must be delivered as “True D/M/E” files. When played back together a True D/M/E will result in an OV Printmaster experience with matching mix levels. The “True” stems will include all associated subtype audio mixed together in a single stem.

    - Example: The True Effects stem will include all subtype audio such as foley and backgrounds.

Dolby Atmos

- A Dolby standard 7.1.4 configuration is the minimum requirement for mix environment.

- Printmasters can have up to 128 input channels of beds and objects. The first 10 tracks are reserved for bed audio.

- All bed channel configurations are acceptable.

Pro Tools Session

- Pro Tools session SMPTE timecode setting must match video frame rate.

- Provide all applicable stems that correspond to the content. Not all stems may apply.

- If Surround Sound does not include an LFE channel then provide a 5.0 track layout.

- Include all applicable bed and objects in their respective Dolby Atmos sessions.

- AS requires a list of plugins, external hardware, and general treatments used in the mix to create any story specific special dialogue effects.

- Include the Domestic Mix Pro Tools session file (.ptx) in the audio package prior to delivering to QC vendor (audio files are not necessary). The Domestic Mix session will be referenced by dubbing mixers to recreate the OV creative intent as accurately as possible.

Track Layouts

| Audio Configuration | Channels |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 7.1 | L | R | C | LFE | Ls | Rs | Lrs | Rrs |
| 5.1 | L | R | C | LFE | Ls | Rs |  |  |
| 5.0 | L | R | C | Ls | Rs |  |  |  |
| DME | C | C | L | R |  |  |  |  |
| 2.0 | Lo | Ro |  |  |  |  |  |  |
| Mono | C |  |  |  |  |  |  |  |

Audio - Locked Cut Split Track Audio

Locked Cut Split Track Audio

- Temp audio may be exported from edit or mix stage.

- Files must be provided as discrete .wav files.

| Asset Spec | Locked Cut Split Track Audio |
| --- | --- |
| File Extension | .wav (discrete for each track) |
| Audio Container | WAV |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Track 1 | Mono Dialogue |
| Track 2 | Mono Effects |
| Track 3 | Stereo Music Left |
| Track 4 | Stereo Music Right |

Audio - Near Field

- Mono and 2.0 audio assets do not require LFE Channel Filtering, as they do not include an LFE channel.

- Program Layout:

SOF EOF

Program

FFOP LFOP

00:00:00:00

Nearfield WAV

| File Type | WAV |
| --- | --- |
| File Extension | .wav |
| Audio Container | WAV |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Monitoring Levels | 79dB - 82dB |
| Loudness Target | -27 LKFS ± 2 LU |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

Near Field Pro Tools Session

| File Type | Pro Tools Session |
| --- | --- |
| File Extension | .ptx (Pro Tools Session) |
| Pro Tools Version | 9.x or later |
| Audio Container | PTX |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Monitoring Levels | 79dB - 82dB |
| Loudness Target | -27 LKFS ± 2 LU |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

Near Field Asset Specs

| Asset Spec | File Type | Track Layouts | Audio Mix/Stem | Notes |
| --- | --- | --- | --- | --- |
| Near Field Printmaster 2.0 | WAV | 2.0 | Printmaster |  |
| Near Field Printmaster 5.1 | WAV | 5.1 | Printmaster |  |
| Near Field Printmaster Session | Pro Tools Session | 5.1/2.0 | Printmaster |  |
| Near Field M&E Session | Pro Tools Session | 5.1/2.0 | M&E |  |
|  |  | 5.1/5.0/ 2.0/Mono | Optionals | If more than one Optional group exists please label as groups as OPT A, OPT B, OPT C. |
|  |  | 2.0 | Dialogue Stem |  |
| Near Field Stems Session | Pro Tools Session | 5.0/2.0 | Dialogue | True Dialogue Stem – See “Audio Asset Specs” above for details |
|  |  | 5.1/2.0 | Music | True Music Stem – See “Audio Asset Specs” above for details |
|  |  | 5.1/2.0 | Music Undipped |  |
|  |  | 5.1/2.0 | Effects | True Effects Stem – See “Audio Asset Specs” above for details |
|  |  | 5.1/2.0 | Fully Filled Effects | Not required for Documentary-Style or Unscripted content. |
|  |  | 5.0/2.0 | Foley |  |
|  |  | 5.0/2.0 | Vocal |  |
|  |  | 5.0/2.0 | Background |  |
|  |  | 5.0/2.0 | Group |  |
|  |  | 5.1/2.0 | Mix Minus Narration | Typically provided for Documentary-Style or Unscripted content. |
|  |  | 5.1/2.0 | Narration |  |
| Near Field Audio Description Session | Pro Tools Session | 2.0 | AD Voice Over |  |
|  |  | 5.1/2.0 | Printmaster |  |
| Near Field Dolby Atmos Printmaster Session | Pro Tools Session | Beds/Objects | Printmaster |  |
| Near Field Dolby Atmos M&E Session | Pro Tools Session | Beds/Objects | M&E |  |
|  |  | Beds/Objects | Optionals | If more than one Optional group exists please label groups as OPT A, OPT B, OPT C. |
|  |  | 2.0 | Dialogue Stem |  |
| Near Field Dolby Atmos Stems Session | Pro Tools Session | Beds/Objects | Dialogue | True Dialogue Stem – See “Audio Asset Specs” above for details |
|  |  | Beds/Objects | Music | True Music Stem – See “Audio Asset Specs” above for details |
|  |  | Beds/Objects | Music Undipped |  |
|  |  | Beds/Objects | Effects | True Effects Stem – See “Audio Asset Specs” above for details |
|  |  | Beds/Objects | Fully Filled Effects | Not required for Documentary-Style or Unscripted content. |
|  |  | Beds/Objects | Foley |  |
|  |  | Beds/Objects | Vocal |  |
|  |  | Beds/Objects | Background |  |
|  |  | Beds/Objects | Group |  |
|  |  | Beds/Objects | Mix Minus Narration | Typically provided for Documentary-Style or Unscripted content. |
|  |  | Beds/Objects | Narration |  |

Near Field Dolby Atmos Printmaster

| Asset Spec | Near Field Dolby Atmos Printmaster |
| --- | --- |
| File Extension | .wav |
| Audio Container | WAV (ADM BWF) |
| Audio Codec | PCM |
| ADM Profile Format | Dolby Atmos Master |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Monitoring Levels | 79dB - 82dB |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

Near Field Audio Description 2.0

| Asset Spec | Near Field Audio Description 2.0 |
| --- | --- |
| File Extension | .mov (does not include video) |
| Audio Container | MOV |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 2 |
| Monitoring Levels | 79dB - 82dB |
| Loudness Target | -27 LKFS ± 2 LU |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Channel Mapping |  |
| Channel 1 | Audio Description Left Only |
| Channel 2 | Audio Description Right Only |

Near Field Audio Description 5.1 and 2.0

| Asset Spec | Near Field Audio Description 5.1 and 2.0 |
| --- | --- |
| File Extension | .mov (does not include video) |
| Audio Container | MOV |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 8 |
| Monitoring Levels | 79dB - 82dB |
| Loudness Target | -27 LKFS ± 2 LU |
| Loudness Scale | 1770-1 with Dialogue Gating exclusion algorithm/Dolby Dialogue Intelligence |
| Maximum True Peak | -2dbTP |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |
| Channel Mapping |  |
| Channel 1 | Audio Description Left |
| Channel 2 | Audio Description Right |
| Channel 3 | Audio Description Center |
| Channel 4 | Audio Description LFE |
| Channel 5 | Audio Description Left Surround |
| Channel 6 | Audio Description Right Surround |
| Channel 7 | Audio Description Left Only |
| Channel 8 | Audio Description Right Only |

Audio - Theatrical

- If 7.1 mix is being made for the title then the applicable mixes and stems should be included in the respective Pro Tools session.

- LFE Channel Filter not required Mono or 2.0 audio.

- Program Layout (24fps Example):

| SOF EOF |  |  |  |  |
| --- | --- | --- | --- | --- |
| Leader | 2 Pop | Program | Black | 2 Pop |
| 8 Sec | 1 Frame | FFOP LFOP | 47 Frames | 1 Frame |
| 01:00:00:00 | 01:00:06:00 | 01:00:08:00 |  |  |

Theatrical WAV

| File Type | WAV |
| --- | --- |
| File Extension | .wav |
| Audio Container | WAV |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Monitoring Levels | 85dB |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |

Theatrical Pro Tools Session

| File Type | Pro Tools Session |
| --- | --- |
| File Extension | .ptx (Pro Tools Session) |
| Pro Tools Version | 9.x or later |
| Audio Container | WAV |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Monitoring Levels | 85dB |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |

Theatrical Asset Specs

| Asset Spec | File Type | Track Layouts | Audio Mix/Stem | Notes |
| --- | --- | --- | --- | --- |
| Theatrical Printmaster 5.1 | WAV | 5.1 | Printmaster |  |
| Theatrical Printmaster 7.1 | WAV | 7.1 | Printmaster |  |
| Theatrical Printmaster Session | Pro Tools Session | 7.1/5.1/2.0 | Printmaster |  |
| Theatrical M&E Session | Pro Tools Session | 7.1/5.1/2.0 | M&E |  |
|  |  | 7.1/5.1/5.0/ 2.0/Mono | Optionals | If more than one Optional group exists please label as groups as OPT A, OPT B, OPT C. |
|  |  | 2.0 | Dialogue Stem |  |
| Theatrical Stems Session | Pro Tools Session | 7.0/5.0/2.0 | Dialogue | True Dialogue Stem – See “Audio Asset Specs” above for details |
|  |  | 7.1/5.1/2.0 | Music | True Music Stem – See “Audio Asset Specs” above for details |
|  |  | 7.1/5.1/2.0 | Music Undipped |  |
|  |  | 7.1/5.1/2.0 | Effects | True Effects Stem – See “Audio Asset Specs” above for details |
|  |  | 7.1/5.1/2.0 | Fully Filled Effects | Not required for Documentary-Style or Unscripted content. |
|  |  | 7.0/5.0/2.0 | Foley |  |
|  |  | 7.0/5.0/2.0 | Vocal |  |
|  |  | 7.0/5.0/2.0 | Background |  |
|  |  | 7.0/5.0/2.0 | Group |  |
|  |  | 7.1/5.1/2.0 | Mix Minus Narration | Typically provided for Documentary-Style or Unscripted content. |
|  |  | 7.1/5.1/2.0 | Narration |  |
| Theatrical Audio Description Session | Pro Tools Session | Mono | AD Voice Over |  |
| Theatrical Dolby Atmos Printmaster Session | Pro Tools Session | Beds/Objects | Printmaster |  |
| Theatrical Dolby Atmos M&E Session | Pro Tools Session | Beds/Objects | M&E |  |
|  |  | Beds/Objects | Optionals | If more than one Optional group exists please label as groups as OPT A, OPT B, OPT C. |
|  |  | 2.0 | Dialogue Stem |  |
| Theatrical Dolby Atmos Stems Session | Pro Tools Session | Beds/Objects | Dialogue | True Dialogue Stem – See “Audio Asset Specs” above for details |
|  |  | Beds/Objects | Music | True Music Stem – See “Audio Asset Specs” above for details |
|  |  | Beds/Objects | Music Undipped |  |
|  |  | Beds/Objects | Effects | True Effects Stem – See “Audio Asset Specs” above for details |
|  |  | Beds/Objects | Fully Filled Effects | Not required for Documentary-Style or Unscripted content. |
|  |  | Beds/Objects | Foley |  |
|  |  | Beds/Objects | Vocal |  |
|  |  | Beds/Objects | Background |  |
|  |  | Beds/Objects | Group |  |
|  |  | 2.0 | Dialogue Stem |  |
|  |  | Beds/Objects | Mix Minus Narration | Typically provided for Documentary-Style or Unscripted content. |
|  |  | Beds/Objects | Narration |  |

Theatrical Dolby Atmos Printmaster

| Asset Spec | Theatrical Dolby Atmos Printmaster |
| --- | --- |
| File Extension | .mxf |
| Audio Container | MXF |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Monitoring Levels | 85db |
| LFE Channel Filter | Low-Pass filter @ 120Hz 24 dB/octave |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |

Theatrical Audio Description

| Asset Spec | Theatrical Audio Description |
| --- | --- |
| File Extension | .wav |
| Audio Container | WAV |
| Audio Codec | PCM |
| Audio Sample Rate | 48kHz |
| Audio Bit Depth | 24-bit |
| Audio Bit Rate | Uncompressed |
| Audio Channels | 1 |
| Monitoring Levels | 85db |
| Start of File | 01:00:00:00 |
| First Frame of Program | 01:00:08:00 |
| Channel Mapping |  |
| Channel 1 | Audio Description Voice Over |

Subtitles

- Link to: SDH and CC Style Guide

- Find Language Specific Time Text Guidelines here.

Forced Narrative, Full SDH, Sans Forced Narrative SDH Subtitle, and Full Subtitles - ITT

- Specification also applies to asset spec Full Subtitle - English Creative Review - ITT.

| Asset Spec | Forced Narrative Subtitle - ITT Full SDH Subtitle - ITT Sans Forced Narrative SDH Subtitle - ITT Full Subtitle - ITT Full Subtitle - English Creative Review - ITT |
| --- | --- |
| File Extension | .itt |
| Timed Text Format | TTML |
| Encoding | UTF-8 |
| Timebase | SMPTE |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Max Lines | 3 |
| Max Characters | 42 |
| Minimum Text Duration | 5/6 of a second (20 frames for 24 fps) |
| Maximum Text Duration | 8 seconds per subtitle event |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

Forced Narrative, Full SDH, Sans Forced Narrative SDH, and Full Subtitle - CAP

- Only required for Japanese language titles.

| Asset Spec | Forced Narrative Subtitle - CAP Sans Forced Narrative SDH - CAP Full SDH Subtitle - CAP |
| --- | --- |
| File Extension | .cap |
| Timed Text Format | CAP (Videotron Lambda) |
| Encoding | Shift-JIS, UTF-8 BOM, or UTF-16 BOM |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Max Lines - Dialogue | 2 |
| Max Lines - Forced Narratives | 3 |
| Max Characters - Horizontal | 13 |
| Max Characters - Vertical | 11 |
| Minimum Text Duration | 1 second |
| Maximum Text Duration | 7 seconds per subtitle event |
| Start of File | 00:00:00:00 |
| First Frame of Program | 00:00:00:00 |

Metadata

Note: Provide either HDR Metadata or HDR Metadata Report, not both. HDR Metadata Report (.pdf) is preferred.

HDR Metadata

- Required per episode.

- Any white on-screen text or graphics should not exceed peak luminance of 1000 nits.

- In/out points for calculation set to include full program, including logos and credits, all of which should have the same Mastering Display Primaries and Color Volume.

HDR Metadata Report

- Generated by tools like Colorfront Trankoder, MTI Cortex, etc.

- Match image aspect ratio, resolution, and frame rate of program.

  - If the image aspect ratio changes from letterbox to full frame, use the framing value "full frame".

- Include values for:

  - Max Fall

  - Max CLL

  - Max Luminance

| Asset Spec | HDR Report |
| --- | --- |
| File Extension | .pdf |
| Color Space | Rec.2020 |
| HDR Gamut Target | P3 |
| Max Luminance | 1000 Nits |

HDR-10 Metadata

- Whole numbers can only be used.

- Needs to be calculated from the image sequence as specified by Consumer Technology Association (CTA) 861-H (2020).

| Asset Spec | HDR-10 Metadata |
| --- | --- |
| File Extension | .txt |
| SMPTE Standard | ST 2086:2018 - Mastering Display Color Volume Metadata Supporting High Luminance and Wide Color Gamut Images |
| Format | G(#, #) B(#, #) R(#, #) WP(#, #) L(Peak Luminance, Min Luminance) --max-cll [MaxCLL],[MaxFALL] |
| Example | G(13250, 34500) B(7500, 3000) R(34000, 16000) WP(15635, 16450) L(10000000, 0) --max-cll (507,357) |

Dolby Vision Metadata

| Asset Spec | Dolby Vision Metadata Dolby Vision Textless Element Reel Metadata |
| --- | --- |
| File Extension | .xml |
| Dolby Vision Metadata | CMv4.0 |
| Color Encoding Primaries | Rec.2020 |
| Metadata Version | 4.0.2 XML |
| Level 1 – (min, avg, max) | Required per shot |
| Level 3 - L1 Offset Metadata | Optional |
| Level 6 - MaxCLL/MaxFALL | MaxCLL = 0 MaxFALL = 0 |
| Level 8 - Target Trim Metadata | Required for 1000 nit and 100 nit trims. |
| Level 254 - Content Mapping Version | 4 1 (Indicates latest backwards backward compatible algorithm) |
| Frame Rate | 23.98, 24, 25, 29.97, 30, 50, 59.94, 60 |
| Canvas Aspect Ratio | 1.77778 |
| Image Aspect Ratio | Project dependent |

Dolby Vision Metadata Example

<DolbyLabsMDF xmlns="http://www.dolby.com/schemas/dvmd/4_0_2">
<Version>4.0.2</Version>

<CanvasAspectRatio>1.77778</CanvasAspectRatio>
<ImageAspectRatio>2.4</ImageAspectRatio>

<Rate>
<n>24000</n>
<d>1001</d>
</Rate>

<ColorEncoding>
<Primaries>
<Red>0.708 0.292</Red>
<Green>0.17 0.797</Green>
<Blue>0.131 0.046</Blue>
</Primaries>
<WhitePoint>0.3127 0.329</WhitePoint>
<PeakBrightness>10000</PeakBrightness>
<MinimumBrightness>0</MinimumBrightness>
<Encoding>pq</Encoding>
<ColorSpace>rgb</ColorSpace>
<SignalRange>computer</SignalRange>
</ColorEncoding>

<MasteringDisplay>
<ID>20</ID>
<Name>1000-nit, P3, D65, ST.2084 (PQ), Full</Name>
<Primaries>
<Red>0.68 0.32</Red>
<Green>0.265 0.69</Green>
<Blue>0.15 0.06</Blue>
</Primaries>
<WhitePoint>0.3127 0.329</WhitePoint>
<PeakBrightness>1000</PeakBrightness>
<MinimumBrightness>0.0001</MinimumBrightness>
<DiagonalSize>42</DiagonalSize>
</MasteringDisplay>

<Level254 level="254">
<DMMode>0</DMMode>
<DMVersion>2</DMVersion>
<CMVersion>4 1</CMVersion>
</Level254>

Script Deliverables

- Acceptable file formats are .docx, .fdx, and .pdf.

Production Scripts

Production Script

Script provided by production team prior to completing post production. Not a representation of the final program. Final Draft (.fdx) files are preferred for Production Scripts.

Production Script (Pivot)

This is a translation from the original version (OV) language production script to a pivot language for localization purposes. Script provided by production team prior to completing post production. Not a representation of the final program. Final Draft (.fdx) files are preferred for Production Scripts.

Scripts

As Broadcast Script

Script that matches final program. Timecode will match longplay files.

As Broadcast Script - Theatrical

Script that matches final program. Timecode will match DCP which is in reels.

Audio Description Script

Audio Description Script

Transcript of Audio Description voice over track.

Documentation

AMS Post contact will advise which documents are required for each Production. Not all documents may be required.

All Paper Deliverables should be uploaded to Box.com. For access to the titles Box folders please reach out to your AMS Post Executive.

| Documentation | Description |
| --- | --- |
| Approved Credits | A copy of the main and end titles as they appear in the final cut of the picture in Microsoft Word and PDF format. The version needs to be clean (no guild markings or watermarks). All logos included in the credits should be placed in the document as they appear on screen. If the Picture is in a non-English language, English translations of the main titles, main on ends, and end crawl must be provided in a Word or Excel doc. |
| Cast/Talent/ Personnel Agreements; Minors | Copies of fully-executed long form agreements (including nudity riders, if applicable) for all personnel and entities who are afforded credit on-screen in the main (or main-on-end) titles or the billing block. All talent agreements must be freely assignable and include work-for-hire or copyright assignment language, a grant of rights to use the talent’s name, voice and likeness and waiver of talent’s rights to injunctive relief. MINORS: All agreements for all minors appearing in the Picture shall be provided. The Work Permits (if applicable in the jurisdiction in which principal production took place), Guardian Release Forms (if applicable), Parental Consent/Inducement Agreements, Production Permits (e.g., Permit To Employ Minors as required in the jurisdictions in which principal production took place), the Trust Account documents (if applicable in the jurisdiction in which principal production took place (e.g., “Coogan Account” for U.S. Productions)), etc. shall accompany all agreements for all minors. All documents shall be fully executed. All Talent Agreements for all minors with a principal role or a speaking role or those minors appearing in a scene with excessive and/or graphic violence, profanity, nudity, sexual activity or other potentially offensive material shall be confirmed by a California court or a court in the jurisdiction in which principal photography took place. In the event that court confirmation is not available in the jurisdiction in which principal photography took place, a letter from an attorney stating that the agreements are valid, binding and enforceable under the laws of the jurisdiction shall be provided. |
| Character Deletion/Addition Report | This document tracks any actors or characters that have been cut from or added to the final version of each episode |
| Clip Reuse Form | The Clip Reuse form tracks all reused material used in each episode. |
| Dailies Report | The camera reports of raw unedited camera footage that was shot in one day of production. |
| Final Continuity Sheet | This document includes all the details from a show to ensure consistency from shot to shot and scene to scene. Not needed for unscripted titles. |
| Final Production & Post Production Contact Sheet | Final list of all cast, crew members, post production team members, vendors and key stakeholders. |
| Font Memo | A document that indicates all fonts utilized throughout the program. This memo will serve as a glossary for new, created fonts and commonly used fonts . It's also important for maintaining a consistent look for international localization. Production Legal requires font licensing terms for fonts purchased from subscription sites to be submitted to them for approval. |
| Inventory Manifest - Audio | Manifest of all Audio assets delivered to Studios for archive for production to verify to ensure full receipt of delivered materials. |
| Inventory Manifest - Digital Source Master | Manifest of all DSM assets delivered to Studios for archive for production to verify to ensure full receipt of delivered materials. |
| Inventory Manifest - Final Archival Elements | Need to define what is included. |
| Inventory Manifest - Non-graded Archival Master | Manifest of all NAM assets delivered to Studios for archive for production to verify to ensure full receipt of delivered materials. |
| Lab Access Letter | Physical Delivery of Laboratory Access letters, in the form to be mutually agreed upon by the parties, must give AS joint access to all negative (physical or digital) and duplicating materials listed therein and must be executed by the production company/lead studio and by each laboratory in possession of any and all of the materials specified in the Co-Production or Acquisition Agreement. |
| Licensed Stock Footage & Still Photos Log | Log of all signed licenses for all stock footage, images, photographs, clips, artwork, books, copyrighted and/or trademarked materials, or other third-party materials used in the Picture. |
| Other Agreements | Access to all other agreements and documents relating to the Picture (e.g., employment agreements, guild earning reports, budget top sheets, negative cost statements, clearances, releases, location agreements, minor agreements (permits/parental consent forms, guardian release forms, production permits, trust account documents, etc.), nudity riders, any other documentation related to minors and, if any payments are due in connection with such agreements, evidence of any such payments in full, etc.) as may be required. |
| Post Milestone Calendar | A spreadsheet that tracks key deliverable dates, including but not limited to: Post Start, Studio Cut 1, Studio Cut 2, Locked Timing Cut, Final Mix, Delivery for QC and Final Delivery. The PMC must be updated/submitted/uploaded to Box weekly every Friday through completion of final episode delivery. |
| Post Production Schedule | A detailed schedule that includes production and post production activities. It must be in a monthly long-form calendar format that is clearly labeled with creation dates/version numbers, and must be updated every Friday through completion for final episode delivery. |
| Post Vendor Sheet | A sheet that tracks all vendors utilized for the show. |
| Producer's Errors and Omissions Insurance | E&O certificate in accordance with the terms of the Agreement. A copy of the E&O insurance application, E&O policy and endorsement in accordance with the terms of the Agreement must also be included. In the event that any material featured in the Picture is relying upon the fair use doctrine, a copy of the legal opinion from an approved vendor shall also be provided. |
| Releases | To the extent that long form agreements do not exist for any person identified by name, voice or likeness in the Picture, then signed releases from those individuals delivered digitally. |
| Workflow Diagram and Technical Info | A document details capture, dailies & post production workflow. |
| Cast List | A list of all cast members appearing in a show. |
| Change List - Final | A timecode specific breakdown of changes made within the Final Production Master. |
| Change List - Locked Cut | A timecode specific breakdown of changes made within the locked cut. |
| Cuts and Dailies Distribution List | A comprehensive list of AS and show personnel for cuts/dailies distribution. |
| Delivery Checklist | Final Master Delivery Checklist - this checklist is included in the Post Doc bundle that AS provides to show executives. The checklist needs to be filled out and submitted for every episode delivered and should be used as a guide to confirm that all materials have been submitted for each delivery. |
| Final File Names | List of all approved video, timed text, and audio description files required by MAM team for distribution and archive purposes. Files must follow file naming conventions. |
| Lower Thirds/Chyron Spotting List | A list that should include all text within each episode excluding credit beds and MT sequences. Episodics should have one spreadsheet per episode. |
| Shot List - DCDM Textless Element Reel | List of all textless shots included in the DCDM textless element reel. |
| Shot List - HD SDR Textless Element Reel | List of all textless shots included in the HD SDR textless element reel. |
| Shot List - UHD Dolby Vision Textless Element Reel | List of all textless shots included in the UHD Dolby Vision textless element reel. |
| Shot List - UHD HDR Textless Element Reel | List of all textless shots included in the UHD HDR textless element reel. |
| Shot List - UHD SDR Textless Element Reel | List of all textless shots included in the HD SDR textless element reel. |
| Dubbing Restrictions | Document to indicate what limitations for in-territory dubbing. |
| Glossary of Key Names and Phrases | A comprehensive document that includes a column of key names and phrases in the original content language, as well as translations for each term into other languages in corresponding columns. Translations are required for every language in which translations already exist or are especially crucial. |
| Music Cue Sheet | Cue sheets must be submitted in the English language showing the particulars of all music (songs and score) contained in the Episode delivered, including the title of each composition, and with respect to each composition: (i) the names of composers; (ii) the names of publishers (including percentage of ownership if more than one publisher); (iii) the usages (whether background instrumental, background vocal, etc.); (iv) the place for each composition showing the in/out timecode within the footage and total duration for each cue; and the (v) performing rights society involved. If any composition is within the public domain, Producer shall identify such composition(s) on the cue sheet with a publishing affiliation of “Public Domain.” In the event that any third-party material includes music (whether score and/or source), such music shall also be identified on the Music Cue Sheet. All music cue sheets must include the Amazon Originals Logo (TOP) logo music cue as the first cue the Amazon Studios Logo (END) as the last cue. |
| Music Licenses and Documentation | Commercial music licensing documents are included in the Music Docs bundle. These include: quote requests/confirmation letters/sync licenses/music licensing reports. |
| Music Rights Contract Agreements | Contracts required for all music key hires including: composer, music supervisor, music producer, original songwriter, and work-for-hire deals. All music key hires must be reviewed and approved in writing by AS Music before they are offered a position. |
| Vanacore Music Library License Agreement - For Unscripted only | A fully-executed Master/Sync Agreement with Vanacore Music Library indicating that the use of music in the Production falls under Amazon's overall blanket agreement (if any Vanacore music cues were used). |
| Vocal Lead Sheet | Required for original music compositions. |
| X-Ray Music List | A track list of all licensed commercial music that includes song title, artist name, in/out timecodes. |
| PSE Test | Harding test is an automatic test for photosensitive epilepsy (PSE) provocative image sequences in television content. |
| 2257 and 2257A Regulations | If the Picture includes any depiction of simulated sexually explicit conduct: a copy of the certification to the U.S. Attorney General under 18 U.S.C §2257A(h) in the form required by 28 C.F.R. §75.9 (a “Certification”) applicable to the Picture from the certifying Licensor of the Picture; and with respect to each performer (including minor performers) in the Picture, identifiable information for each such performer (including the performer’s name, addresses and date of birth). |
| Affidavit | An affidavit sworn to by an officer of the production company stating that all costs of production have been paid and there are no liens, encumbrances or claims as of the date of the Affidavit. |
| American Humane Association Disclaimer | United States: If the Picture contains the American Humane Association (AHA) disclaimer (“no animals were harmed in the making of this film”), Licensor must provide a copy of the disclaimer approval letter issued by the AHA. International: Regions outside of the US must provide a comparable certification. As stipulated in the credit policy section of the GPM, include the following message translated in OV language “All animals on this production were treated with the utmost respect and care.” |
| Chain of Title | Chain of title documents include Underlying Rights Documents, Copyright Registration Certificate, Certificate of Authorship, Certificate of Origin, Short Form License/Instrument of Transfer, Title Report, Copyright Report, Copyright Mortgage and Assignment, UCC-1 Financing Statement, and UCC Search. |
| Dolby/Sound License | An executed copy of the license agreement for use of Dolby sound, or any sound license in connection with the Picture, for the duration of the license term and for unlimited distribution. If a Sound License was not necessary for completion of the Picture, a statement noting the release of such obligations must be provided. |
| DVD/Blu-Ray Soundtrack Statement of Obligations | A statement outlining the DVD, Blu-Ray, soundtrack or similar obligations (including, without limitation, all obligations pertaining to the number of copies to be issued, etc.) for all individuals affiliated with the Picture. If there are no DVD, Blu-Ray, soundtrack or similar obligations, Licensor must provide a written statement to that effect, upon which Amazon may rely. |
| Editing Restrictions | A “cut and paste” compilation of any and all contractual rights or obligations (whether by contract or by union affiliation) to any party regarding supervising, consulting or participating in cutting/editing for airlines or television, or other post-production or cutting/editing processes. |
| Fact Sheet | Template provided by Amazon Studios post to Licensor. |
| Guild Approvals | Guild Approval documents include Directors Guild Credit Approval, Writers Guild Verification, and Production Designer Credit Waiver. |
| Main Title Credit Statement | A “cut and paste” compilation of all contractual main title on-screen credit obligations from the applicable agreements for all cast, crew and entities who have been accorded main title credit on-screen. |
| MPA Rating Certificate | Paid rating certificate from the Motion Picture Association. with a restriction not greater than “R”. |
| Paid Advertising Approvals/Restrictions/ Obligations Statement | Paid Advertising Approvals/Restrictions/Obligations Statement - A “cut and paste” compilation of any and all contractual (a) paid advertising credit obligations; (b) name and likeness and other advertising obligations; (c) restrictions on use of name and likeness; and (d) any consultation and approval rights (including any tie-in obligations) over advertising. |
| Premiere and Film Festival Obligations | Digital Cinema Format. |
| Residuals Documentation | All residuals documentation necessary for Amazon to meet residuals obligations, including but not limited to the following: Residuals Worksheet, SAG/AFTRA, IATSE, and American Federation of Musicians. |
| Subordinations | Subordinations in form acceptable to Amazon executed by all parties providing financing and completion bonds for the production and delivery of the Picture or having any lien, charge, or security interest in the Picture or its revenues |

Archival

All original assets used in the title must be delivered to AMS at wrap of Post Production with corresponding checksum report (.MD5 preferred). All archival files must be stored uncompressed in their original format and shall not be packaged as compressed archives (e.g., .zip, .rar, .tar).

Original Capture Files (OCF)

- Consult with your Post Executive/Delivery Manager if you want to deliver a file format not noted below.

| Asset Spec | Original Capture Files |
| --- | --- |
| File Extensions | .dpx, .exr, .tiff, .mxf, .axr |
| Resolution | Camera Native Resolution |
| Color Space | Camera Native Color Space |
| Chroma Bit Depth | 16-bit (preferred), 12-bit, 10-bit |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Start of File | Frame 0 |

Image Sequence

Non-Graded Archival Master (NAM)

- Image sequence of the non-color graded final program.

  - Uncompressed image sequences required.

  - Match bit depth of best original camera source.

  - Match the Production Master from Start of File (SOF) to End of File (EOF).

  - Includes all final visual effects.

  - Must be Textless. Doesn’t include Forced Narrative subtitles, titles, or graphic overlays.

  - A framing chart must be provided with each NAM file.

  - Can be delivered in reels or longplay.

  - Un-matted and in Digital Intermediate (DI) working resolution.

    - If delivering in reels, image sequences must be delivered in individual folders for each reel.

  - Require delivery of 16-bit .exr files when working in Academy Color Encoding System (ACES).

    - Export files following ST 2065-1:2021 - SMPTE Standard - Academy Color Encoding Specification (ACES)

    - AP0 color primaries and white point.

    - Transfer Function is Linear.

| Asset Spec | Non-Graded Archival Master |
| --- | --- |
| File Extensions | .dpx, .exr, .tiff |
| Resolution | Digital Intermediate (DI) Working Resolution |
| Chroma Bit Depth | 16-bit (preferred), 10-bit |
| Chroma Sub Sampling | 4:4:4 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Start of File | Frame 0 |

Digital Source Masters

- Uncompressed image sequences required.

- Match the Production Master from Start of File (SOF) to End of File (EOF).

- If a program contains Forced Narratives, deliver a Non-Subtitled Digital Source Master.

- Includes final color correction, visual effects, titles, and graphics overlays.

Movies

- Can be delivered in reels or longplay.

  - If delivering in reels, image sequences must be delivered in individual folders for each reel.

UHD HDR Digital Source Master

| Asset Spec | UHD HDR Digital Source Master UHD HDR Digital Source Master Textless Element Reel |
| --- | --- |
| File Extensions | .tiff |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Color Space | Color encoding primaries: Rec.2020 Color gamut: P3 D65 or Rec.2020 |
| Max Luminance | 1000 |
| Chroma Bit Depth | 16-bit |
| Chroma Sub Sampling | 4:4:4 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Start of File | Frame 0 |

UHD SDR Digital Source Master

- If a title is delivering only in Rec.709 deliver as 10-bit .dpx files.

| Asset Spec | UHD SDR Digital Source Master UHD SDR Digital Source Master Textless Element Reel |
| --- | --- |
| File Extensions | .dpx, .tiff |
| Resolution Height | 2160 |
| Resolution Width | 3840 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Chroma Bit Depth | 16-bit (preferred), 10-bit |
| Max Luminance | 100 |
| Chroma Sub Sampling | 4:4:4 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Start of File | Frame 0 |

HD SDR Digital Source Master

- If a title is delivering only in Rec.709 deliver as 10-bit .dpx files.

| Asset Spec | HD SDR Digital Source Master HD SDR Digital Source Master Textless Element Reel |
| --- | --- |
| File Extensions | .dpx, .tiff |
| Resolution Height | 1080 |
| Resolution Width | 1920 |
| Display Aspect Ratio | 16x9 |
| Color Space | Rec.709 |
| Chroma Bit Depth | 16-bit (preferred), 10-bit |
| Max Luminance | 100 |
| Chroma Sub Sampling | 4:4:4 |
| Frame Rate | 23.976, 24, 25, 29.97, 30, 48, 50, 59.94, 60 |
| Start of File | Frame 0 |

4K Flat DCI-P3 Digital Source Master

| Asset Spec | 4K Flat DCI-P3 Digital Source Master 4K Flat DCI-P3 Digital Source Master Textless Element Reel |
| --- | --- |
| File Extensions | .dpx, .exr, .tiff |
| Resolution Height | 2160 |
| Resolution Width | 3996 |
| Display Aspect Ratio | Flat (1.85) |
| Color Space | DCI-P3 |
| Chroma Bit Depth | 16-bit (preferred), 10-bit |
| Chroma Sub Sampling | 4:4:4 |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Start of File | Frame 0 |

4K Scope DCI-P3 Digital Source Master

| Asset Spec | 4K Scope DCI-P3 Digital Source Master 4K Scope DCI-P3 Digital Source Master Textless Element Reel |
| --- | --- |
| File Extensions | .dpx, .exr, .tiff |
| Resolution Height | 1716 |
| Resolution Width | 4096 |
| Display Aspect Ratio | Scope (2.39) |
| Color Space | DCI-P3 |
| Chroma Bit Depth | 16-bit (preferred), 10-bit |
| Chroma Sub Sampling | 4:4:4 |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Start of File | Frame 0 |

2K Flat DCI-P3 Digital Source Master

| Asset Spec | 2K Flat DCI-P3 Digital Source Master 2K Flat DCI-P3 Digital Source Master Textless Element Reel |
| --- | --- |
| File Extensions | .dpx, .exr, .tiff |
| Resolution Height | 1080 |
| Resolution Width | 1998 |
| Display Aspect Ratio | Flat (1.85) |
| Color Space | DCI-P3 |
| Chroma Bit Depth | 16-bit (preferred), 10-bit |
| Chroma Sub Sampling | 4:4:4 |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Start of File | Frame 0 |

2K Scope DCI-P3 Digital Source Master

| Asset Spec | 2K Scope DCI-P3 Digital Source Master 2K Scope DCI-P3 Digital Source Master Textless Element Reel |
| --- | --- |
| File Extensions | .dpx, .exr, .tiff |
| Resolution Height | 858 |
| Resolution Width | 2048 |
| Display Aspect Ratio | Scope (2.39) |
| Color Space | DCI-P3 |
| Chroma Bit Depth | 16-bit (preferred), 10-bit |
| Chroma Sub Sampling | 4:4:4 |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Start of File | Frame 0 |

Digital Source Master Textless Element Reels

- Match Digital Source Master specs.

- Textless Elements are to be provided in sequential order, as they pertain to the program.

- In/out points to be set on hard cuts, where no dissolves or transitions are present.

- See Textless Element Reels for more info.

- Textless Elements must be frame accurate to Production Master.

  - Example: If the Production Master first Textless Element is at 00:02:36:12, provide textless shot at 00:00:10:12 on the Textless Element Reel.

  - The timecodes will not be the same but the frame numbers will be.

  - Black between textless shots will be between 2-3 seconds due to the frame requirement.

Digital Cinema Distribution Masters (DCDM)

4K Flat Digital Cinema Distribution Master

| Asset Spec | 4K Flat Digital Cinema Distribution Master 4K Flat Digital Cinema Distribution Master Textless Element Reel |
| --- | --- |
| File Extensions | .tiff |
| Resolution Height | 2160 |
| Resolution Width | 3996 |
| Display Aspect Ratio | Flat (1.85) |
| Color Space | XYZ |
| Chroma Bit Depth | 16-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | Gamma 2.6 |
| Frame Rate | 24 |
| Start of File | Frame 0 |

4K Scope Digital Cinema Distribution Master

| Asset Spec | 4K Scope Digital Cinema Distribution Master 4K Scope Digital Cinema Distribution Master Textless Element Reel |
| --- | --- |
| File Extensions | .tiff |
| Resolution Height | 1716 |
| Resolution Width | 4096 |
| Display Aspect Ratio | Flat (1.85) |
| Color Space | XYZ |
| Chroma Bit Depth | 16-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | Gamma 2.6 |
| Frame Rate | 24 |
| Start of File | Frame 0 |

2K Flat Digital Cinema Distribution Master

| Asset Spec | 2K Flat Digital Cinema Distribution Master 2K Flat Digital Cinema Distribution Master Textless Element Reel |
| --- | --- |
| File Extensions | .tiff |
| Resolution Height | 1080 |
| Resolution Width | 1998 |
| Display Aspect Ratio | Flat (1.85) |
| Color Space | XYZ |
| Chroma Bit Depth | 16-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | Gamma 2.6 |
| Frame Rate | 24 |
| Start of File | Frame 0 |

2K Scope Digital Cinema Distribution Master

| Asset Spec | 2K Scope Digital Cinema Distribution Master 2K Scope Digital Cinema Distribution Master Textless Element Reel |
| --- | --- |
| File Extensions | .tiff |
| Resolution Height | 858 |
| Resolution Width | 2048 |
| Display Aspect Ratio | Scope (2.39) |
| Color Space | XYZ |
| Chroma Bit Depth | 16-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | Gamma 2.6 |
| Frame Rate | 24 |
| Start of File | Frame 0 |

Dolby Vision Digital Cinema Distribution Masters (DCDM)

4K Flat Dolby Vision Digital Cinema Distribution Master

| Asset Spec | 4K Flat Dolby Vision Digital Cinema Distribution Master 4K Flat Dolby Cinema Digital Cinema Distribution Master Textless Element Reel |
| --- | --- |
| File Extensions | .tiff |
| Resolution Height | 2160 |
| Resolution Width | 3996 |
| Display Aspect Ratio | Flat (1.85) |
| Color Space | XYZ |
| Chroma Bit Depth | 16-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Start of File | Frame 0 |

4K Scope Dolby Vision Digital Cinema Distribution Master

| Asset Spec | 4K Scope Dolby Vision Digital Cinema Distribution Master 4K Scope Dolby Cinema Digital Cinema Distribution Master Textless Element Reel |
| --- | --- |
| File Extensions | .tiff |
| Resolution Height | 1716 |
| Resolution Width | 4096 |
| Display Aspect Ratio | Scope (2.39) |
| Color Space | XYZ |
| Chroma Bit Depth | 16-bit |
| Chroma Sub Sampling | 4:4:4 |
| Transfer Function | ST.2084 (PQ) |
| Frame Rate | 24, 25, 30, 48, 50, 60 |
| Start of File | Frame 0 |

Digital Cinema Distribution Masters Textless Element Reels

- Match Digital Cinema Distribution Masters specs.

- Textless Elements are to be provided in sequential order, as they pertain to the program.

- In/out points to be set on hard cuts, where no dissolves or transitions are present.

- See Textless Element Reels for more info.

- Textless Elements must be frame accurate to Production Master.

  - Example: If the Production Master first Textless Element is at 00:02:36:12, provide a textless shot at 00:00:10:12 on the Textless Element Reel.

  - The timecodes will not be the same, but the frame numbers will be.

  - Black between textless shots will be between 2-3 seconds due to the frame requirement.

Press Enter to type after or press Shift + Enter to type before the widget

Saved